Used to switch to the “Overclocking – Basic” page.  The user can make performance modifications dynamically from this page, which take effect during the current session.

Used to switch to the “Overclocking – Advanced” page.  The user can make performance and specific BIOS modifications that take effect upon the next boot from this page.

Used to switch to the “Information” page.  This page displays in-depth system hardware and software revision and capability details.

This button will apply all changes made and exit the application

Note: Changes made to the “Overclocking – Basic” page only apply to the current session.  To apply these changes for subsequent boots, or to save them as a profile, the Save Profile button must be used.

Cancel all changes made (set back to defaults) and exit the application.

This button allows the user to apply all changes made.  After the values are applied the application will remain open at the current active tab.

Note: Changes made to the “Overclocking – Basic” page only apply to the current session.  To apply these changes for subsequent boots, or to save them as a profile, the Save Profile button must be used.

Cancel all changes made (set back to defaults) and exit the application.

Load values from a saved profile and applies them.

Save settings in a profile.

Note: “Overclocking – Basic” page changes may be selected to be enabled for each boot.  Boot profiles are saved in the system registry, and establish changes at boot of the OS.  The user will be given the chance to disable the profile at each boot.

Toggle the performance-tracking pane to view progressive temperature, voltage, and bus status.

Provides clocking control of specified bus speeds in the group. The adjustments can be made using the mouse to drag the slider, or by tabbing to slider and using the arrow keys.  Note: The maximum overclock capability is +25% of the nominal frequency, while the underclocking is limited to –50% of the nominal frequency.

Note: When overclocking the system, it may be desirable to increase the voltage level of the components to give more operating margin.  Also, For higher Front Side Bus and Memory Bus adjustments, it is recommended that the memory timings be increase, or relaxed, to provide better stability.

Note: Selecting to overvoltage or overclock a component may void its warranty due to exceeding recommended specifications.   NVIDIA and the board manufacturer are not responsible for damage that may occur when components have tolerances exceeded.

Front Side Bus Speed: For best performance, NVIDIA requires the Front Side Bus and Memory Bus be in synchronous mode; therefore, adjustments to the Front Side Bus make identical adjustments to the Memory Bus.

AGP Bus Speed: Adjusts the external AGP Bus frequency.

Provides control of essential memory timings, which are necessary to adjust stability when overclocking the Front Side Bus and Memory Bus.

Row Address Strobe. This field specifies the row active time.  This is the minimum number of cycles between an activate command and a pre-charge command to the same bank.  This setting has a pull-down menu that allows steps from 1 (most aggressive) to 15 (most relaxed).

RAS to CAS access. This field specifies the minimum number of cycles between an activate command and a read command to the same bank. This setting has a pull-down menu that allows steps from 1 (most aggressive) to 7 (most relaxed).

Memory Bank Switch. This field specifies the row pre-charge time.  This is the minimum number of cycles between a pre-charge command and an activate command to the same bank.  This setting has a pull-down menu that allows steps from 1 (most aggressive) to 7 (most relaxed).

Provides overvoltage control of specified voltages in the group. When overclocking the system, it may be desirable to increase the voltage level of the components to give more operating margin.

Note: Selecting to over-voltage a component may void its warranty due to exceeding recommended specifications.   NVIDIA and the board manufacturer are not responsible for damage that may occur when components have voltage tolerances exceeded.

# The CPU supply voltage may be adjusted from the operating level of 1.1v to 1.85v.  This adjustment is linear and can be made in step increments of 0.025v.  The adjusting slide bar may be manipulated using the mouse to drag the slider, or by tabbing to the slider and using the arrow keys.

The voltage supply to the system chipset may be adjusted using the pull-down menu.  1.5v is the operating level, but the user my also select 1.6v and 1.7v from the pull-down menu.

# The voltage supply to the DIMMs may be adjusted using the pull-down menu.  2.6v is the operating level, but the user my also select 2.7v and 2.8v from the pull-down menu.

# The auxiliary voltage supply may be adjusted using the pull-down menu.  1.6v is the operating level, but the user my also select 1.7v from the pull-down menu.

# The voltage supply for the AGP bus may be adjusted using the pull-down menu.  1.5v is the operating level, but the user my also select 1.6v and 1.7v from the pull-down menu.

Provides control of the CPU and secondary fan.  It is not recommended that these fans be disabled or adjusted down, unless adequate cooling measures have been taken.  NVIDIA and the board manufacturer are not responsible for damage resultant from thermal loads due to manipulation of the fan speed.

Allows select of “Off” to “Max” with range steps defined by the fan installed.  The adjustments can be made using the mouse to drag the slider, or by tabbing to slider and using the arrow keys.

Allows select of “Off” to “Max” with range steps defined by the fan installed.  The adjustments can be made using the mouse to drag the slider, or by tabbing to slider and using the arrow keys.

Selects the priority order of boot devices.  Changes take effect upon the next boot and remain unchanged for subsequent boots unless a CMOS default condition is caused.

Defines the primary boot device.

Defines the secondary boot device.

Defines the final boot device.

Used to select predefined performance settings or enter an Expert mode to select parameters. Changes take effect upon the next boot and remain unchanged for subsequent boots unless a CMOS default condition is caused.

Optimal: Standard operating settings are used.  Most stable selection.

Aggressive: Settings are adjusted to enhance performance, but most systems should remain stable.

Turbo: Settings are adjusted to highest performance capabilities.  Because of chipset, memory, processor, and motherboard differences; this may not be a stable setting.

Expert: Opens other performance options to be manipulated and override predefined settings.

Sets the clock multiplier used by the CPU.

Row Address Strobe. This field specifies the row active time.  This is the minimum number of cycles between an activate command and a pre-charge command to the same bank.  This setting has a pull-down menu that allows steps from 1 (most aggressive) to 15 (most relaxed).

RAS to CAS access. This field specifies the minimum number of cycles between an activate command and a read command to the same bank. This setting has a pull-down menu that allows steps from 1 (most aggressive) to 7 (most relaxed).

Memory Bank Switch. This field specifies the row pre-charge time.  This is the minimum number of cycles between a pre-charge command and an activate command to the same bank.  This setting has a pull-down menu that allows steps from 1 (most aggressive) to 7 (most relaxed).

Overrides the CAS latency value reported by the installed DIMMs.

Provides overvoltage control of specified voltages in the group. When overclocking the system, it may be desirable to increase the voltage level of the components to give more operating margin.  Changes take effect upon the next boot and remain unchanged for subsequent boots unless a CMOS default condition is caused.

Note: Selecting to over-voltage a component may void its warranty due to exceeding recommended specifications.   NVIDIA and the board manufacturer are not responsible for damage that may occur when components have voltage tolerances exceeded.

# The CPU supply voltage may be adjusted from the operating level of 1.1v to 1.85v.  This adjustment is linear and can be made in step increments of 0.025v.  The adjusting slide bar may be manipulated using the mouse to drag the slider, or by tabbing to the slider and using the arrow keys.

The voltage supply to the system chipset may be adjusted using the pull-down menu.  1.5v is the operating level, but the user my also select 1.6v and 1.7v from the pull-down menu.

# The voltage supply to the DIMMs may be adjusted using the pull-down menu.  2.6v is the operating level, but the user my also select 2.7v and 2.8v from the pull-down menu.

# The auxiliary voltage supply may be adjusted using the pull-down menu.  1.6v is the operating level, but the user my also select 1.7v from the pull-down menu.

# The voltage supply for the AGP bus may be adjusted using the pull-down menu.  1.5v is the operating level, but the user my also select 1.6v and 1.7v from the pull-down menu.

Adjust miscellaneous system performance settings.  Changes take effect upon the next boot and remain unchanged for subsequent boots unless a CMOS default condition is caused.

Used to enable or disable the Internal CPU cache.  For best performance, this should remain enabled.

Used to enable or disable the External CPU cache.  For best performance, this should remain enabled.

Used to set the aperture size for AGP.

Used to enable or disable SSE/SSE2 instructions.

Used to enable or disable AGP 8x support for the external AGP device.

Used to enable or disable AGP 2.0 speed for the external AGP device.

Used to enable or disable AGP 3.0 speed for the external AGP device

Used to set the HyperTransport ™ speed.

Provides the brand, name, family, model, and stepping information for the installed processor. Selecting the “More” button provides additional in-depth information for the processor.

Provides calculated system performance information for the bus and CPU speeds.

Note: Front side and memory bus speeds are expressed as DDR.

Provides details regarding the motherboard model and PCB version.

Provides details regarding the chipset model and silicon version.

Provides version numbers and file dates for currently installed NVIDIA chipset and graphics drivers.  Provides versions details for the OS and DirectX ™ versions.

Provides the vendor name and part number for each installed DIMM.  Selecting the “More” button provides additional in-depth information for each DIMM.

Sets the polling interval for the temperature, voltage, and speed tracking graphs.

Displays the current CPU fan speed.

Displays a timeline chart of the CPU die or base temperature and System temperature at a rate defined by the “Update” pull-down box.

Displays a graph of the current voltage levels and bus speeds sampled at a rate defined by the “Update” pull-down box.