NVIDIA NVSystemUtility BIOS Interface

CONFIDENTIAL INFORMATION
DO NOT COPY

Version 1.00
September 16, 2003

Document Change History

| Version | Date | Owner | Reason for Change |
| --- | --- | --- | --- |
| 0.1 | 03/18/03 | Y. Wan | Initial version 0.1 |
| 0.2 | 03/19/03 | Y. Wan | Some function codes are changed. Added functions related to 3 more voltage control, 2 more temperature control, and 2 more fan control for OEM use. Reserved some function code for future use, Changed interface version number from 1 byte to 2 bytes. Removed function code that enables voltage margin OUTEN. Used 8 bytes more in scratch register from 4488h to 448Fh to return description string for each get function. Changed fan speed return value to a DWORD. |
| 0.3 | 03/24/03 | Y. Wan | Changed Function code that is used to generate software SMI from A0h to C8h. Fixed some mismatch sub-function code. |
| 0.4 | 04/28/03 | Y. Wan | Clarified temperature is returned in one signed byte and is in Celsius. Change CPU VID to CPU Voltage. Indicated the hard coded 4400h is actually got from PCI configuration space. |
| 0.5 | 05/20/03 | Y. Wan | Changed interface for transfer CMOS data. |
| 0.6 | 06/04/03 | D. Hunt | Add Source code examples for functions. Examples are from reference implementation on the PCB 180-7R012-0000-D00 CRB. |
| 0.7 | 06/06/03 | D. Hunt | Added ability for the OEM to define the SMI function code through the MCP Scratch 7 Register. |
| 0.8 | 06/18/03 | D. Hunt | Added FSB Frequency to Get CMOS Location 0 (sub-function 27h). Added ability to notify no OEM functions are supported to sub-function 01h. |
| 0.9 | 08/05/03 | D. Hunt | Removed FSB Frequency CMOS assignments. Added sub-function for reading the actual voltage levels. Added returns for temperature thresholds. Added CPU Multiplier CMOS select. Added sub-function to support motherboard vendor, revision, and model reporting. |
| 1.0 | 9/16/03 | D. Hunt | Added sub-function 02Ah for reporting DIMM SPD address and position information to the NVSU application. |

Table of Contents

1.0	Introduction	1

1.1	NVSystemUtility System BIOS Interface	1

1.1.1	Description of Communication	1

1.1.2	Status Codes	1

1.1.3	Description String	2

1.1.4	Basic Sub-functions	2

1.1.4.1	Sub-function code 01h	2

1.1.4.2	Sub-function code 02h	3

1.1.4.3	Sub-function code 03h	3

1.1.4.4	Sub-function code 04h	4

1.1.4.5	Sub-function code 05h	4

1.1.4.6	Sub-function code 06h	4

1.1.4.7	Sub-function code 07h	4

1.1.4.8	Sub-function code 08h	5

1.1.4.9	Sub-function code 09h	5

1.1.4.10	Sub-function code 12h	5

1.1.4.11	Sub-function code 13h	5

1.1.4.12	Sub-function code 16h	6

1.1.4.13	Sub-function code 17h	6

1.1.4.14	Sub-function code 18h	6

1.1.4.15	Sub-function code 19h	7

1.1.4.16	Sub-function code 1Ah	7

1.1.4.17	Sub-function code 1Bh	7

1.1.4.18	Sub-function code 1Ch	8

1.1.4.19	Sub-function code 1Dh	8

1.1.4.20	Sub-function code 1Eh	8

1.1.4.21	Sub-function code 1Fh	9

1.1.4.22	Sub-function code 24h	9

1.1.4.23	Sub-function code 25h	10

1.1.4.24	Sub-function code 26h	11

1.1.4.25	Sub-function code 27h	11

1.1.4.26	Sub-function code 28h	12

1.1.4.27	Sub-function code 2Ah	13

1.1.5	OEM Defined Sub-functions	14

1.1.5.1	Sub-function code 0Ah	14

1.1.5.2	Sub-function code 0Bh	14

1.1.5.3	Sub-function code 0Ch	14

1.1.5.4	Sub-function code 0Dh	14

1.1.5.5	Sub-function code 0Eh	14

1.1.5.6	Sub-function code 0Fh	15

1.1.5.7	Sub-function code 10h	15

1.1.5.8	Sub-function code 11h	15

1.1.5.9	Sub-function code 14h	15

1.1.5.10	Sub-function code 15h	15

1.1.5.11	Sub-function code 20h	15

1.1.5.12	Sub-function code 21h	16

1.1.5.13	Sub-function code 22h	16

1.1.5.14	Sub-function code 23h	16

2.0	Sample Code	17

2.1	Tables	17

2.1.1	Function Table For The SMI Handler	17

2.1.2	Voltage Tables	19

2.1.2.1	CPU Voltage Table	19

2.1.2.2	Chipset Core Voltage Table	20

2.1.2.3	AGP Voltage Table	20

2.1.2.4	Memory Voltage Table	20

2.1.2.5	Auxiliary Voltage Table	20

2.1.3	Voltage GPIO Control Tables	21

2.1.3.1	DefaultVoltageMarginTable	21

2.1.3.2	CPU Voltage Margin GPIO Table	23

2.2	Entry and Data Handling	24

2.2.1	Defining The SMI Function Code	24

2.2.2	Main SMI Entry	24

2.2.3	Data Handling	26

2.2.3.1	Get Input Data	26

2.2.3.2	Set Return Data	26

2.3	Support Functions	27

2.3.1	DefaultFunc:	27

2.3.2	GetInterfaceVersion:	27

2.3.3	EnableHWMonitor	28

2.3.4	CS_I2CReadByte	29

2.3.5	CS_I2CwriteByte	30

2.3.6	CS_Chk_SMBus_READY	31

2.3.7	SuperIORegRead	32

2.3.8	SuperIORegWrite	32

2.4	Temperature Functions	33

2.4.1	GetCPUTemperature	33

2.4.2	GetMotherBoardTemperature	34

2.5	Voltage Functions	35

2.5.1	Voltage Support Functions	35

2.5.1.1	EnableVoltageMargin	35

2.5.1.2	EnableDefaultVoltMargin	36

2.5.1.3	GetVoltMarginOutenStatus	37

2.5.1.4	EnableVIDOverrideCtrl	38

2.5.1.5	GetVIDOverrideCtrlStatus	39

2.5.1.6	SetOverrideVoltage	40

2.5.1.7	GetOverrideGPIOVoltage	41

2.5.1.8	FindVoltageInTable	42

2.5.2	Auxiliary Voltage	43

2.5.2.1	GetAuxVoltage	43

2.5.2.2	SetAuxVoltage	44

2.5.3	Memory Voltage	45

2.5.3.1	GetMemoryVoltage	45

2.5.3.2	SetMemoryVoltage	46

2.5.4	AGP Voltage	47

2.5.4.1	GetAGPVoltage	47

2.5.4.2	SetAGPVoltage	48

2.5.5	Chipset Core Voltage	49

2.5.5.1	GetCSCoreVoltage	49

2.5.5.2	SetCSCoreVoltage	50

2.5.6	CPU Voltage	51

2.5.6.1	SetCPUVoltage	51

2.5.6.2	GetCPUVoltage	52

2.6	Fan Functions	53

2.6.1	CPU Fan Functions	53

2.6.1.1	GetCPUFanSpeed	53

2.6.1.2	GetCPUFanPower	55

2.6.1.3	SetCPUFanPower	56

2.6.2	System Fan Functions	57

2.6.2.1	GetSecondFanSpeed	57

2.6.2.2	GetSecondFanPower	59

2.6.2.3	SetSecondFanPower	60

2.7	CMOS Functions	61

2.7.1	GetCMOSLocation0	61

2.7.2	GetCMOSLocation1	63

2.7.3	WPTComputeCMOSCheckSumAll	64

2.7.4	WPTComputeCMOSCheckSum	65

2.7.5	Motherboard Information	66

2.7.5.1	ReadMBInfo	66

2.7.5.2	GetBoardVersion	67

2.8	Source Definitions	67

# Introduction

## NVSystemUtility System BIOS Interface

To support the NVIDIA NVSystemUtility, the System BIOS must provide a set of functions in the software SMI handler.  To invoke these functions, driver will generate software SMI with a function code defined by the OEM in the MCP Scratch Register 7 bit(s) [31:24].  This should be defined at the end of POST.  For the purpose of the provided examples, C8h is used.

During POST, system control registers of the Media Communications Processor are mapped to an IO base address that can be read from bus 0, device 1, function 0, offset 64h-65h.  To ensure software SMI is enabled, registers offset 05h bit2 and offset 04h bit0 needs to be 1.  In general, these bits should already be set by BIOS during POST.

### Description of Communication

Writing function code C8h to register offset 2Eh will generate software SMI related to NVSystemUtility events.  Register offset 2Fh will be used for input sub-function code and output status code.  Currently code from 01h to 27h is used for sub-functions.  After a SMI is handled, status code SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register-offset 2Fh. The driver will check register offset 2Fh for status each time after a SMI is generated and handled.   Scratch registers offset 84h-8Fh are used to pass data in and out during SMI handling.  Among these scratch registers, offset 88h to 8Fh are always used to pass a description string to driver when an OEM GET function is called.  Driver needs to save the value of these scratch registers before using them and restore them after the SMI is handled.  Interrupt needs to be disabled before issuing a SMI to avoid potential data loss.

### Status Codes

- Status Code (returned in register offset 2Fh)

| SUCCESS | 00h : Sub-function execution succeeds. |
| --- | --- |
| INVALID_FUNCTION | FFh: Returned if sub-function code is not within range of 01h to 27h. |
| FAILURE | FEh : Sub-function execution fails. |

### Description String

Only sub-function 0A, 0B, 0E, 0F, 14, 15, 20, 22, and 24 will return description string in registers.  Driver will use description string directly without any modification.

### Basic Sub-functions

NOTE: For the following sub-functions the System Control IO Base is assumed to be 4400h.

#### Sub-function code 01h

Purpose: Get the NVSystemUtility interface version.

| Code | Description |
| --- | --- |
| Out 442Fh, 01h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In ax, 4484h | Returned interface version will be saved in register 4484h and 4485h. Register 4485h is major version and Register 4484h is minor version. So 0100h means version 1.0 |

The most significant word of this sub-function is used as an indication for OEM function support.  If is read as 0001h, the OEM function is supported.  Otherwise, no OEM function support from BIOS.  Corresponding code change in BIOS is as following:

OEMSUPPORT 	equ 		00 ;Not supported

in 	sub function 01:

mov 	ecx, (OEMSUPPORT shl 16) + WPT_INTERFACE_VERSION

#### Sub-function code 02h

Purpose: Get CPU Fan Speed

| Code | Description |
| --- | --- |
| Out 442Fh, 02h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4484h | Returned fan speed in RPM as an DWORD will be saved in register 4484h. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 03h

Purpose: Get CPU Fan Power

| Code | Description |
| --- | --- |
| Out 442Fh, 03h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In al, 4484h | Returned fan power will be saved in register 4484h. Value will be one byte between 00 to FF. FF means full power on. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 04h

Purpose: Set CPU Fan Power

| Code | Description |
| --- | --- |
| Out 442Fh, 04h | Sub-function code |
| Out 4484h, al | Fan power Value between 00 to FF is given to 4484h. |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 05h

Reserved.

#### Sub-function code 06h

Purpose: Get System Fan Speed

| Code | Description |
| --- | --- |
| Out 442Fh, 06h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4484h | Returned fan speed in RPM as an DWORD will be saved in register 4484h. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 07h

Purpose: Get System Fan Power

| Code | Description |
| --- | --- |
| Out 442Fh, 07h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In al, 4484h | Returned fan power will be saved in register 4484h. Value will be one byte between 00 to FF. FF means full power on. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 08h

Purpose: Set System Fan Power

| Code | Description |
| --- | --- |
| Out 442Fh, 08h | Sub-function code |
| Out 4484h, al | Fan power Value between 00 to FF is given to 4484h. |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 09h

Reserved

#### Sub-function code 12h

Purpose: Get CPU Temperature

| Code | Description |
| --- | --- |
| Out 442Fh, 12h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

Data is returned in three bytes.

Byte0 = Temperature as one signed byte in Celsius.

Byte1 = Temperature threshold for over-temp condition.

Byte2 = Bit 0: 0 = CPU Die temperature reported, 1 = CPU Case temperature reported.

#### Sub-function code 13h

Purpose: Get Motherboard Temperature

| Code | Description |
| --- | --- |
| Out 442Fh, 13h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

Data is returned in three bytes.

Byte0 = Temperature as one signed byte in Celsius.

Byte1 = Temperature threshold for over-temp condition.

#### Sub-function code 16h

Purpose: Get CPU Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 16h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4484h | Returned voltage will be saved in registers from 4484h to 4487h. The format is single precision floating point format. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 17h

Purpose: Set CPU Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 17h | Sub-function code |
| Out 4484, eax | CPU Voltage is given in single precision floating point format. |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 18h

Purpose: Get Chipset Core Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 18h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4484h | Returned voltage will be saved in registers from 4484h to 4487h. The format is single precision floating point format. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 19h

Purpose: Set Chipset Core Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 19h | Sub-function code |
| Out 4484, eax | CPU Voltage is given in single precision floating point format. |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 1Ah

Purpose: Get AGP Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 1Ah | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4484h | Returned voltage will be saved in registers from 4484h to 4487h. The format is single precision floating point format. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 1Bh

Purpose: Set AGP Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 1Bh | Sub-function code |
| Out 4484, eax | CPU Voltage is given in single precision floating point format. |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 1Ch

Purpose: Get Memory Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 1Ch | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4484h | Returned voltage will be saved in registers from 4484h to 4487h. The format is single precision floating point format. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 1Dh

Purpose: Set Memory Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 1Dh | Sub-function code |
| Out 4484, eax | CPU Voltage is given in single precision floating point format. |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 1Eh

Purpose: Get AUX Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 1Eh | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4484h | Returned voltage will be saved in registers from 4484h to 4487h. The format is single precision floating point format. |
| In eax, 4488h | Returned description string. |
| In edx, 4489h | Returned description string. |

#### Sub-function code 1Fh

Purpose: Set AUX Voltage

| Code | Description |
| --- | --- |
| Out 442Fh, 1Fh | Sub-function code |
| Out 4484, eax | CPU Voltage is given in single precision floating point format. |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 24h

Purpose: Get Actual Voltage Reading

| Code | Description |
| --- | --- |
| Out 442Fh, 24h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

Data will be returned in single precision floating point format in the following MCP Scratch registers.

| Scratch Register | Format | Data Returned |
| --- | --- | --- |
| 1 | dword | CPU actual voltage reading |
| 2 | dword | Chipset actual voltage reading |
| 3 | dword | AGP core actual voltage reading |
| 4 | dword | Memory actual voltage reading |
| 5 | dword | Auxiliary actual voltage reading |

#### Sub-function code 25h

| Code | Description |
| --- | --- |
| Out 442Fh, 24h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

The motherboard vendor ID is returned in MCP Scratch register 1 bits [15:0] (see Motherboard Vendor ID table).  Bits [31:16] of the register are used to reflect the board version (ex. 0100h = v1.0).

Motherboard Vendor ID Table

| ID Number | Vendor Name |
| --- | --- |
| 0x00 | Not Supported |
| 0x01 | NVIDIA |
| 0x02 | Asus |
| 0x03 | MSI |
| 0x04 | Abit |
| 0x05 | Chaintech |
| 0x06 | Epox |
| 0x07 | Leadtek |
| 0x08 | Shuttle |
| 0x09 | Gigabyte |
| 0x0A | FIC |
| 0x0B | Aopen |
| 0x0C | Soltek |
| 0x0D | Biostar |
| 0x0E | Wistron |
| 0x0F | Mitac |
| 0x10 | DFI |
| 0x11 | Albatron |
| 0x12 | Jetway |
| 0x13 | Surwin |
| 0x14 | QDI/Legend |
| 0x15 | Pine |
| 0x16 | SOYO |
| 0x17 | Arima |

The MCP Scratch registers 2 through 7 will be used for the Mother board model.  Each byte represents the Hex equivalent of an ASCII character.  Any register bytes not used should be populated with 0.

#### Sub-function code 26h

Purpose: Recalculate CMOS checksum.

| Code | Description |
| --- | --- |
| Out 442Fh, 26h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |

#### Sub-function code 27h

Purpose: Get CMOS Location 0.

| Code | Description |
| --- | --- |
| Out 442Fh, 27h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4480h | Returned data will be saved in scratch registers from 4480h to 449fh |

Data format:

|  | Bits 0-7 | Bits 8-15 | Bits 16-23 | Bits 24-31 |
| --- | --- | --- | --- | --- |
| Scratch register0 (4480h) | System performance CMOS offset | System performance cmos bitmap | CPU interface cmos offset | CPU interface cmos bitmap |
| Scratch register1 (4484h) | Memory timing mode cmos offset | Memory timing mode cmos bitmap | tRas cmos offset | tRas cmos bitmap |
| Scratch register2 (4488h) | tRcd CMOS offset | tRcd CMOS bitmap | tRp CMOS offset | tRp CMOS bitmap |
| Scratch register3 (448Ch) | Cas latency CMOS offset | Cas latency CMOS bitmap | AGP 8X support CMOS offset | AGP 8X support CMOS bitmap |
| Scratch register4 (4490h) | First boot device CMOS offset | First boot device CMOS bitmap | Second boot device CMOS offset | Second boot device CMOS bitmap |
| Scratch register5 (4494h) | Third boot device CMOS offset | Third boot device CMOS bitmap | AGP aperture size CMOS offset | AGP aperture size CMOS bitmap |
| Scratch register6 (4498h) | CPU internal cache CMOS offset | CPU internal cache CMOS bitmap | CPU external cache CMOS offset | CPU external cache CMOS bitmap |
| Scratch register7 (449Ch) | SSE/SSE2 instruction CMOS offset | SSE/SSE2 instruction CMOS bitmap | CPU Multiplier CMOS offset | CPU Multiplier CMOS bitmap |

Note:  If CMOS data field is not support, a 0FFh will be returned for CMOS offset and 0 will be returned for CMOS bitmap.

#### Sub-function code 28h

Purpose: Get CMOS Location 1.

| Code | Description |
| --- | --- |
| Out 442Fh, 28h | Sub-function code |
| Out 442Eh, 0C8h | Software SMI function code |
| In al, 442Fh | Either SUCCESS, INVALID_FUNCTIONCODE, or FAILURE will be returned in register 442Fh. |
| In eax, 4480h | Returned data will be saved in scratch registers from 4480h to 449fh |

Data format:

|  | Bits 0-7 | Bits 8-15 | Bits 16-23 | Bits 24-31 |
| --- | --- | --- | --- | --- |
| Scratch register0 (4480h) | AGP 2.0 Speed CMOS offset | AGP 2.0 Speed CMOS bitmap | AGP 3.0 Speed CMOS offset | AGP 3.0 Speed CMOS bitmap |
| Scratch register1 (4484h) | LDT Speed CMOS offset | LDT Speed CMOS bitmap | Reserved | Reserved |
| Scratch register2 (4488h) | Over-voltage Aux CMOS offset | Over-voltage Aux CMOS bitmap | Over-voltage core CMOS offset | Over-voltage core CMOS bitmap |
| Scratch register3 (448Ch) | Over-voltage AGP CMOS offset | Over-voltage AGP CMOS bitmap | Over-voltage memory CMOS offset | Over-voltage memory CMOS bitmap |
| Scratch register4 (4490h) | Over-voltage CPU VID CMOS offset | Over-voltage CPU VID CMOS bitmap | Reserved | Reserved |
| Scratch register5 (4494h) | Reserved | Reserved | Reserved | Reserved |
| Scratch register6 (4498h) | Reserved | Reserved | Reserved | Reserved |
| Scratch register7 (449Ch) | Reserved | Reserved | Reserved | Reserved |

Note:  If CMOS data field is not support, a 0FFh will be returned for CMOS offset and 0 will be returned for CMOS bitmap.

#### Sub-function code 2Ah

Function code 2Ah for GetDimmAddrInfo:

In: None.

Out: Scratch Register 0-8 saved DimmAddrInfo as follows.

The address information for each DIMM will be described by 1 word-length data.  The maximum number of supported DIMMs is 16. Each Scratch register contains address information for two DIMMs.

1 Word-length address info data format is as following:

Bit 7:0	is the SMBus address.

Bit 11:8 is the corresponding DIMM slot number on the motherboard silk screen.

Bit 15:12 is the SMBus segment (Used if more than 8 DIMM slots available - most likely in multiprocessor platform.)

DIMM information should be filled start from scratch register 0, then 1, 2, etc. and from least significant word to most significant word.  Filling sequence should be sorted according to DIMM slot number. SMBus address portion should be set to 0 for unused scratch registers.  SMBus segment portion should be 0 until MP support is added in the NVSU application.  Once a 0 is encountered in a scratch register for the SMBus address portion, then the rest of scratch registers are ignored.

Example: 5 Dimms slot in system.

DIMM	SMBus address

0	51h

1	53h

2	54h

3	50h

4	55h

Resulting address info in scratch registers.

Scratch register 0: 	0153 0051h

Scratch register 1:	0350 0254h

Scratch register 2:	0000 0455h

Scratch register 3:	0000 0000h

Scratch register 4:	0000 0000h

Scratch register 5:	0000 0000h

Scratch register 6:	0000 0000h

Scratch register 7:	0000 0000h

If this function is not supported, The NVIDIA System Utility will scan all the memory SMBus addresses to get the info, and the memory slot number will not be displayed.

### OEM Defined Sub-functions

Following sub-functions are used for OEM additions.  Returned description string needs to describe what the control is.  If not implemented, return INVALID_FUNCTIONCODE in status register.

Note: These Sub-functions may not by used by the NVSystemUtility until the 2nd generation release.

#### Sub-function code 0Ah

Purpose: Get Fan Speed

Definition same as above Get CPU fan Speed (See Sub-function 02h)

#### Sub-function code 0Bh

Purpose: Get Fan Power

Definition same as above Get CPU fan Power (See Sub-function 03h)

#### Sub-function code 0Ch

Purpose: Set Fan Power

Definition same as above Set CPU fan Power (See Sub-function 04h)

#### Sub-function code 0Dh

Reserved

#### Sub-function code 0Eh

Purpose: Get Fan Speed

Definition same as above Get CPU fan Speed (See Sub-function 02h)

#### Sub-function code 0Fh

Purpose: Get Fan Power

Definition same as above Get CPU fan Power (See Sub-function 03h)

#### Sub-function code 10h

Purpose: Set Fan Power

Definition same as above Set CPU fan Power (See Sub-function 04h)

#### Sub-function code 11h

Reserved

#### Sub-function code 14h

Purpose: Get OEM1 Temperature

Definition same as above Get CPU Temperature (See Sub-function 12h)

#### Sub-function code 15h

Purpose: Get OEM2 Temperature

Definition same as above Get CPU Temperature (See Sub-function 12h)

#### Sub-function code 20h

Purpose: Get OEM1 voltage

Definition same as above Get CPU Voltage (See Sub-function 16h)

#### Sub-function code 21h

Purpose: Set OEM1 voltage

Definition same as above Set CPU Voltage (See Sub-function 17h)

#### Sub-function code 22h

Purpose: Get OEM2 voltage

Definition same as above Get CPU Voltage (See Sub-function 16h)

#### Sub-function code 23h

Purpose: Set OEM2 voltage

Definition same as above Set CPU Voltage (See Sub-function 17h)

# Sample Code

The Sample code provided is written for the NVIDIA Reference board (PCB 180-7R012-0000-D00); therefore, it may require changes dependent upon the specific OEM hardware design.

## Tables

### Function Table For The SMI Handler

public WPTFunctionTable

WPTFunctionTable:

;Function Offset                                                        	 	Function code

dw      (offset CS:GetInterfaceVersion - offset CS:SMI_Handler_Start)		;01h

dw      (offset CS:GetCPUFanSpeed - offset CS:SMI_Handler_Start)              	;02h

dw      (offset CS:GetCPUFanPower - offset CS:SMI_Handler_Start)                ;03h

dw      (offset CS:SetCPUFanPower - offset CS:SMI_Handler_Start)                 ;04h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;05h

dw      (offset CS:GetSecondFanSpeed - offset CS:SMI_Handler_Start)             ;06h

dw      (offset CS:GetSecondFanPower - offset CS:SMI_Handler_Start)            ;07h

dw      (offset CS:SetSecondFanPower - offset CS:SMI_Handler_Start)             ;08h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;09h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;0Ah

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;0Bh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;0Ch

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;0Dh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;0Eh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;0Fh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;10h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;11h

dw      (offset CS:GetCPUTemperature - offset CS:SMI_Handler_Start)           	;12h

dw      (offset CS:GetMotherBoardTemperature - offset CS:SMI_Handler_Start) ;13h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;14h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;15h

dw      (offset CS:GetCPUVoltage - offset CS:SMI_Handler_Start)                    ;16h

ifdef K8_CPU_SUPPORT

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;17h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;18h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;19h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;1Ah

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;1Bh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;1Ch

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;1Dh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;1Eh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;1Fh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;20h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;21h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;22h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;23h

else

dw      (offset CS:SetCPUVoltage - offset CS:SMI_Handler_Start)                 	;17h

dw      (offset CS:GetCSCoreVoltage - offset CS:SMI_Handler_Start)               	;18h

dw      (offset CS:SetCSCoreVoltage - offset CS:SMI_Handler_Start)                 ;19h

dw      (offset CS:GetAGPVoltage - offset CS:SMI_Handler_Start)                    ;1Ah

dw      (offset CS:SetAGPVoltage - offset CS:SMI_Handler_Start)                     ;1Bh

dw      (offset CS:GetMemoryVoltage - offset CS:SMI_Handler_Start)              	;1Ch

dw      (offset CS:SetMemoryVoltage - offset CS:SMI_Handler_Start)               	;1Dh

dw      (offset CS:GetAuxVoltage - offset CS:SMI_Handler_Start)                     ;1Eh

dw      (offset CS:SetAuxVoltage - offset CS:SMI_Handler_Start)                      ;1Fh

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;20h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;21h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;22h

dw      (offset CS:DefaultFunc - offset CS:SMI_Handler_Start)                        	;23h

dw      (offset CS:ReadActualVoltage - offset CS:SMI_Handler_Start)               	;24h

dw      (offset CS:ReadMBInfo - offset CS:SMI_Handler_Start)                        	;25h

endif   ;K8_CPU_SUPPORT

dw      (offset CS:WPTComputeCMOSCheckSumAll - offset CS:SMI_Handler_Start) ;26h

dw      (offset CS:GetCMOSLocation0 - offset CS:SMI_Handler_Start)          	;27h

dw      (offset CS:GetCMOSLocation1 - offset CS:SMI_Handler_Start)             ;28h

WPTFunctionTableLen       EQU     ($ - WPTFunctionTable)

### Voltage Tables

#### CPU Voltage Table

public  CPUVoltageTable

CPUVoltageTable:

dd      1.850                   ;1850mV     equ     00h

dd      1.825                   ;1825mV     equ     01h

dd      1.800                   ;1800mV     equ     02h

dd      1.775                   ;1775mV     equ     03h

dd      1.750                   ;1750mV     equ     04h

dd      1.725                   ;1725mV     equ     05h

dd      1.700                   ;1700mV     equ     06h

dd      1.675                   ;1675mV     equ     07h

dd      1.650                   ;1650mV     equ     08h

dd      1.625                   ;1625mV     equ     09h

dd      1.600                   ;1600mV     equ     0Ah

dd      1.575                   ;1575mV     equ     0Bh

dd      1.550                   ;1550mV     equ     0Ch

dd      1.525                   ;1525mV     equ     0Dh

dd      1.500                   ;1500mV     equ     0Eh

dd      1.475                   ;1475mV     equ     0Fh

dd      1.450                   ;1450mV     equ     10h

dd      1.425                   ;1425mV     equ     11h

dd      1.400                   ;1400mV     equ     12h

dd      1.375                   ;1375mV     equ     13h

dd      1.350                   ;1350mV     equ     14h

dd      1.325                   ;1325mV     equ     15h

dd      1.300                   ;1300mV     equ     16h

dd      1.275                   ;1275mV     equ     17h

dd      1.250                   ;1250mV     equ     18h

dd      1.225                   ;1225mV     equ     19h

dd      1.200                   ;1200mV     equ     1Ah

dd      1.175                   ;1175mV     equ     1Bh

dd      1.150                   ;1150mV     equ     1Ch

dd      1.125                   ;1125mV     equ     1Dh

dd      1.100                   ;1100mV     equ     1Eh

dd      VOLTAGE_TABLE_END       ;End OF table

#### Chipset Core Voltage Table

public  CSCoreVoltageTable

CSCoreVoltageTable:

dd      1.70               	;1.70v          00b

dd      1.60               	;1.60v          01b

dd      1.60               	;1.60v          02b

dd      1.50		;Default	;1.50v          03b

dd      VOLTAGE_TABLE_END       ;End OF table

#### AGP Voltage Table

public  AGPVoltageTable

AGPVoltageTable:

dd      1.70               	;1.70v          00b

dd      1.60               	;1.60v          01b

dd      1.60               	;1.60v          02b

dd      1.50 		;Default	;1.50v          03b

dd      VOLTAGE_TABLE_END       ;End OF table

#### Memory Voltage Table

public  MemoryVoltageTable

MemoryVoltageTable:

dd      2.80               	;2.80v          00b

dd      2.70               	;2.70v          01b

dd      2.70              		;2.70v          02b

dd      2.60 		;Default	;2.60v          03b

dd      VOLTAGE_TABLE_END       ;End OF table

#### Auxiliary Voltage Table

public  AuxVoltageTable

AuxVoltageTable:          	;Default is 1.5v

dd      1.60               	;1.60v          00b

dd      1.70               	;1.70v          01b

dd      1.70               	;1.70v          02b

dd      00000000h               	;NULL        03b

dd      VOLTAGE_TABLE_END       ;End OF table

### Voltage GPIO Control Tables

For the examples provided, the GPIO assignments are for a SMSC LPC47M157-NC SIO controller as configured on the NVIDIA C17-SPP and C18D reference platforms.

#### DefaultVoltageMarginTable

The table description is for each GPIO pin that controls voltage margin.

Note: the following table will be within the following table definition.

public  DefaultVoltageMarginTable

DefaultVoltageMarginTable:

; AuxVoltageGPIOTable:

; MemoryVoltageGPIOTable:

; AGPVoltageGPIOTable:

; CSCoreVoltageGPIOTable:

DefaultVoltageMarginTableLen       EQU     ($ - DefaultVoltageMarginTable)

##### Auxiliary Voltage GPIO Table

AuxVoltageGPIOTable:

;Aux Margin1

db      AUX_MARGIN1_GPIO12_CTRL 	; GPIO control register

db      GPIO_CTRL_OUTPUT                	; Set to out function

db      AUX_GPIO_GROUP                  	; GPIO group data register

db      AUX_MARGIN1_DEFAULT_STATUS ; Default Bit status.  Only can be 0 and 1.

db      AUX_MARGIN1_BIT                 	; Bit number for this bit in the GPIO group data reg.

GPIOTableSingleSetLen       EQU     ($ - AuxVoltageGPIOTable)

;Aux Margin2

db      AUX_MARGIN2_GPIO17_CTRL

db      GPIO_CTRL_OUTPUT

db      AUX_GPIO_GROUP

db      AUX_MARGIN2_DEFAULT_STATUS

db      AUX_MARGIN2_BIT

AuxVoltageGPIOTableLen       EQU     ($ - AuxVoltageGPIOTable)

##### Memory Voltage GPIO Table

MemoryVoltageGPIOTable:

;Memory margin1

db      MEM_MARGIN1_GPIO15_CTRL

db      GPIO_CTRL_OUTPUT

db      MEM_GPIO_GROUP

db      MEM_MARGIN1_DEFAULT_STATUS

db      MEM_MARGIN1_BIT

;Memory Margin2

db      MEM_MARGIN2_GPIO16_CTRL

db      GPIO_CTRL_OUTPUT

db      MEM_GPIO_GROUP

db      MEM_MARGIN2_DEFAULT_STATUS

db      MEM_MARGIN2_BIT

MemoryVoltageGPIOTableLen       EQU     ($ - MemoryVoltageGPIOTable)

##### AGP Voltage GPIO Table

AGPVoltageGPIOTable:

;AGP Margin1

db      AGP_MARGIN1_GPIO13_CTRL

db      GPIO_CTRL_OUTPUT

db      AGP_GPIO_GROUP

db      AGP_MARGIN1_DEFAULT_STATUS

db      AGP_MARGIN1_BIT

;AGP Margin2

db      AGP_MARGIN2_GPIO14_CTRL

db      GPIO_CTRL_OUTPUT

db      AGP_GPIO_GROUP

db      AGP_MARGIN2_DEFAULT_STATUS

db      AGP_MARGIN2_BIT

AGPVoltageGPIOTableLen       EQU     ($ - AGPVoltageGPIOTable)

##### Chipset Core Voltage GPIO Table

CSCoreVoltageGPIOTable:

;Chipset core Margin1

db      CSCORE_MARGIN1_GPIO10_CTRL

db      GPIO_CTRL_OUTPUT

db      CSCORE_GPIO_GROUP

db      CSCORE_MARGIN1_DEFAULT_STATUS

db      CSCORE_MARGIN1_BIT

;Chipset Core Margin2

db      CSCORE_MARGIN2_GPIO11_CTRL

db      GPIO_CTRL_OUTPUT

db      CSCORE_GPIO_GROUP

db      CSCORE_MARGIN2_DEFAULT_STATUS

db      CSCORE_MARGIN2_BIT

CSCoreVoltageGPIOTableLen       EQU     ($ - CSCoreVoltageGPIOTable)

db      VOLTMARGINEN_GPIO43_CTRL

db      GPIO_CTRL_OUTPUT

db      VOLTMARGINEN_GPIO_GROUP

db      MARGINOUTEN_ENABLE_STATUS

db      MARGINOUTEN_ENABLE_BIT

#### CPU Voltage Margin GPIO Table

CPUVoltageGPIOTable:

;VID0

db      CPUVID_OVERRIDE0_GPIO25_CTRL

db      GPIO_VID_CTRL_OUTPUT

db      CPUVID_GPIO_GROUP

db      CPUVID_DEFAULT_STATUS

db      CPUVID0_GPIO_BIT

;VID1

db      CPUVID_OVERRIDE1_GPIO26_CTRL

db      GPIO_VID_CTRL_OUTPUT

db      CPUVID_GPIO_GROUP

db      CPUVID_DEFAULT_STATUS

db      CPUVID1_GPIO_BIT

;VID2

db      CPUVID_OVERRIDE2_GPIO20_CTRL

db      GPIO_VID_CTRL_OUTPUT

db      CPUVID_GPIO_GROUP

db      CPUVID_DEFAULT_STATUS

db      CPUVID2_GPIO_BIT

;VID3

db      CPUVID_OVERRIDE3_GPIO21_CTRL

db      GPIO_VID_CTRL_OUTPUT

db      CPUVID_GPIO_GROUP

db      CPUVID_DEFAULT_STATUS

db      CPUVID3_GPIO_BIT

;VID4

db      CPUVID_OVERRIDE4_GPIO22_CTRL

db      GPIO_VID_CTRL_OUTPUT

db      CPUVID_GPIO_GROUP

db      CPUVID_DEFAULT_STATUS

db      CPUVID4_GPIO_BIT

CPUVoltageGPIOTableLen           EQU     ($ - CPUVoltageGPIOTable)

## Entry and Data Handling

### Defining The SMI Function Code

This macro should be used in the end of POST to set correct function code.  The NVIDIA application will get the function code from the MCP Scratch Register 7 bit(s) [31:24] and use it to access the NVSystemUtility functions.

SetWPTFncCode MACRO	;This macro should be used when stack is available.

push    	dx

push    	ax

mov     	dx, SMI_IO_BASE_ADDR + 9Fh       	;Scratch register 7 bit 24-31.

mov     	al, NV_WPT

out     	dx, al

pop     	ax

pop     	dx

ENDM

### Main SMI Entry

This is the main entry for NVSystemUtility interface. Check if it is a NVSystemUtility request and if it is, perform the proper function and return the value.  Data is updated to AL and ECX in SMM state save area.

WPT_SMI_Entry_Main proc near

push    	ax

push    	bx

push    	ecx

; Read input function code and saved in al. Read input data and saved in ecx

call    	GetInputData

cmp     	al, 00h

je      	WPT_SMI_Entry_Main_Invalid     	;invalid function code

cmp     	al, (WPTFunctionTableLen shr 1)

ja      	WPT_SMI_Entry_Main_Invalid      	;invalid function code

mov     	bx, (offset CS:WPTFunctionTable - offset CS:SMI_Handler_Start)

xor     	ah, ah

dec     	al                             			;Calculate correct offset for function

shl     	al, 1

add     	bx, ax

mov     	bx, cs:[bx]

call    	bx                            			; return value in ecx

jc      	WPT_SMI_Entry_Main_Execute_Fail 	; carry set means failed.

mov     	al, WPT_RETURN_SUCCESS

jmp     	WPT_SMI_Entry_Main_Exit

WPT_SMI_Entry_Main_Execute_Fail:

mov     	al, WPT_RETURN_FAIL

jmp     	WPT_SMI_Entry_Main_Exit

WPT_SMI_Entry_Main_Invalid:

mov     	al, WPT_RETURN_INVALID

WPT_SMI_Entry_Main_Exit:

call    	SetReturnData

pop     	ecx

pop     	bx

pop     	ax

ret

WPT_SMI_Entry_Main endp

### Data Handling

#### Get Input Data

Output: AL has the function code and ECX has the data.

GetInputData proc near

push    	dx

mov	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH1_REG

in		eax, dx

mov     	ecx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_STATUS_REG

in      	al, dx

pop     	dx

ret

GetInputData endp

#### Set Return Data

Input: AL has the return code and ECX has the return value; Output: AL and ECX in SMM state save table are updated.

SetReturnData proc near

push    	dx

push   	 si

push    	es

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_STATUS_REG

out     	dx, al

mov    	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH1_REG

mov     	eax, ecx

out    	dx, eax

pop    	es

pop     	si

pop     	dx

ret

SetReturnData endp

## Support Functions

### DefaultFunc:

Output: jump to WPT_SMI_Entry_Main_Invalid label

DefaultFunc proc near

WPTRestoreStack

jmp     	WPT_SMI_Entry_Main_Invalid

DefaultFunc endp

### GetInterfaceVersion:

Output: data is saved in ECX.  Carry clear means success; else failure

GetInterfaceVersion proc near

mov     ecx, WPT_INTERFACE_VERSION

clc

ret

GetInterfaceVersion endp

### EnableHWMonitor

Output: data is saved in ECX.  Carry clear means success; else failure

This is used by the temperature and voltage control functions.

EnableHWMonitor proc near

push    	ax

push   	 bx

mov     	bl, HW_MONITOR_SMBUS_READ_ADDR

mov     	al, SMSC_LPC47M15_SMB_REG_CONFIGURATION

call    	CS_I2cReadByte

jc      	EnableHWMonitorExit

test    	bh, 1

jnz     	EnableHWMonitorExit        ;jump if HWMonitor already enabled.

or      	bh, 21h

mov     	bl, HW_MONITOR_SMBUS_WRITE_ADDR

mov     	al, SMSC_LPC47M15_SMB_REG_CONFIGURATION

call    	CS_I2CWriteByte

EnableHWMonitorExit:

pop    	bx

pop     	ax

ret

EnableHWMonitor endp

### CS_I2CReadByte

This is used only for SMM mode and is used by the temperature and voltage control functions.  Input: BL - 8 bits I2C ID address,AL - byte index. Output: BH - value return if carry cleared, else failed

CS_I2CReadByte  Proc    Near

push    	dx

; Disable stuff in extended control reg.

push    	ax

mov     	al, 00

mov     	dx, SMBUS_PORT2 + 3Eh

out     	dx, al

;Send a not in use to protocol reg.

mov     	dx, SMBUS_PORT2 + 00h

out     	dx, al

pop    	 ax

mov     	ah, al                  ; byte index

mov     	al, bl

mov     	dx, SMBUS_PORT2 + 02h

out     	dx, ax                  ; address and index

; wait SMbus ready

;ROM_CALL Chk_SMBus_READY ;N18

; start I2C read operation

newiodelay

mov     	dx, SMBUS_PORT2 + 00h

mov     	al, 07h                 ; MCP_SMB_PRTCL_PROTOCOL_READ_BYTE

out     	dx, al

newiodelay

; check status OK ?

call    	CS_Chk_SMBus_READY

jc      	CS_I2C_ReadFail          ; SMBus Fail

newiodelay

; read data

mov     	dx, SMBUS_PORT2 + 04h    ; MCP_CONST_SMB1_BASE + MCP_SMB_DATA_00

in      	al, dx

mov     	bh, al                  ; value return

CS_I2C_ReadFail:

pop     	dx

ret

CS_I2CReadByte  Endp

### CS_I2CwriteByte

This is used only for SMM mode and is used by the temperature and voltage control functions.  Input: BL - 8 bits I2C ID address, AL - byte index, BH - value to write.  Output: Fail - if carry set.

CS_I2CWriteByte    Proc    Near

push    	dx

mov     	ah, al                  ; byte index

mov     	al, bl

mov     	dx, SMBUS_PORT2 + 02h

out     	dx, ax                  ; address and index

newiodelay

; put data to SMbus Base + 6

mov     	al, bh

mov     	dx, SMBUS_PORT2 +04h

out     	dx, al                  ; Data0

newiodelay

; wait SMbus ready

call    	CS_Chk_SMBus_READY

; start I2C write operation

newiodelay

mov     	dx, SMBUS_PORT2 + 00h

mov     	al, 06h                 ; MCP_SMB_PRTCL_PROTOCOL_WRITE_BYTE

out     	dx, al

; check status OK ?

call    	CS_Chk_SMBus_READY

pop     	dx

ret

CS_I2CWriteByte    Endp

### CS_Chk_SMBus_READY

This is used only for SMM mode.  Output: Carry set means error and carry clear means ready.

CS_Chk_SMBus_READY      Proc    Near

push    	dx

push    	cx

push    	ax

mov     	dx, SMBUS_PORT2 + 01h     ; Status port

clc

mov     	cx, 2000h

CS_Chk_I2C_OK:

newiodelay

in      	al, dx                   	; get status

or      	al, al                   		; N18

jz      	CS_Chk_I2C_OK         	; not ready

test    	al, 80h                  	; is finished

jnz     	CS_Clear_final

test    	al, 01Fh

jz      	CS_Clear_final

and     	al, 01Fh

cmp     	al, 01Ah             		; SMBus busy?

jne     	CS_SMBus_Err

loop    	CS_Chk_I2C_OK

CS_SMBus_Err:

stc

CS_Clear_final:

out    	dx, al			; clear status

pop     	ax

pop     	cx

pop     	dx

ret

CS_Chk_SMBus_READY      Endp

### SuperIORegRead

Read Super IO registers.  Entry: AH: Register Index.  Exit: AL: the register value read.

SuperIORegRead  proc near

push    	dx

mov     	dx, SUPERIO_GPIO_PORT_BASE

add     	dl, ah

in      	al, dx

pop     	dx

ret

SuperIORegRead endp

### SuperIORegWrite

Read Super IO register.  Entry: AH: Register Index.  AL: Register value

SuperIORegWrite  proc near

push    	dx

mov     	dx, SUPERIO_GPIO_PORT_BASE

add     	dl, ah

out     	dx, al

pop     	dx

ret

SuperIORegWrite endp

## Temperature Functions

Note: For the temperature monitoring, the following part must be removed from the reference board (PCB 180-7R012-0000-D00): U12G2.

### GetCPUTemperature

Reads the CPU die temperature from the CPU internal temperature sensor.  Output: data is saved in ECX.  Carry clear means success, else failure.

GetCPUTemperature proc near

push    	ax

call    	EnableHWMonitor

jc      	GetCPUTemperatureError

mov     	bl, HW_MONITOR_SMBUS_READ_ADDR

mov     	al, CPU_TEMP_SMBUS_OFFSET

call    	CS_I2cReadByte

jc      	GetCPUTemperatureError

movzx   	ecx, bh   ;Save current temperature in ecx

mov     	ch, CPU_TEMP_THRESHOLD

clc

GetCPUTemperatureError:

pop     ax

ret

GetCPUTemperature endp

### GetMotherBoardTemperature

Reads a temperature sensor connected to the SIO on the motherboard.  Output: data is saved in ECX.  Carry clear means success, else failure.

GetMotherBoardTemperature proc near

push    	ax

call    	EnableHWMonitor

jc      	GetMBTemperatureError

mov     	bl, HW_MONITOR_SMBUS_READ_ADDR

mov     	al, BOARD_TEMP_SMBUS_OFFSET

call    	CS_I2cReadByte

jc      	GetMBTemperatureError

movzx   	ecx, bh

mov 	ch, MB_TEMP_THRESHOLD

clc

GetMBTemperatureError:

pop     	ax

ret

GetMotherBoardTemperature endp

## Voltage Functions

### Voltage Support Functions

#### EnableVoltageMargin

Default table loaded.

Entry: SI: table length, DI: table offset, DL: GPIO override status (Bit 0 = 1: override with new data, Bit 0 = 0: don't override), DH: bit 0 is the data for GPIO override if override is enabled.  Exit: DI: is in the end of the table.

EnableVoltageMargin proc near

push    	ax

push    	cx

; Set GPIO to proper status

SetNextGPIOReg:

mov     	ah, cs:[di]

mov     	al, cs:[di+1]

call    	SuperIORegWrite

mov     	ah, cs:[di+2]

call    	SuperIORegRead

mov     	cx, cs:[di+3]

xchg    	ch, cl

ror     	al, cl

and     	al, 0FEh

rol     	al, cl

xchg    	ch, cl

test    	dl, 1              		;Check for override

jz      	NoOverrideVoltage

mov     	cl, ch

shl     	dh, cl

mov     	cl, dh

NoOverrideVoltage:

or      	al, cl

call    	SuperIORegWrite

add     	di, GPIOTableSingleSetLen

sub     	si, GPIOTableSingleSetLen

cmp     	si, 0

jne     	SetNextGPIOReg

pop     	cx

pop     	ax

ret

EnableVoltageMargin endp

#### EnableDefaultVoltMargin

EnableDefaultVoltMargin proc near

push    	si

push    	di

push    	dx

xor     	dx, dx              		;no override for default data

mov     	si, DefaultVoltageMarginTableLen

mov     	di, (offset CS:DefaultVoltageMarginTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	EnableVoltageMargin

clc

pop     	dx

pop     	di

pop     	si

ret

EnableDefaultVoltMargin endp

#### GetVoltMarginOutenStatus

Check status of Volt_margin_out_en.  Exit: carry set if enabled (override enabled)

GetVoltMarginOutenStatus proc near

push    	ax

;check if GPIO as output and GPIO function.

mov     	ah, VOLTMARGINEN_GPIO43_CTRL

call    	SuperIORegRead

and     	al, GPIO_CTRL_OUTPUT_MASK

cmp     	al, 0

jne     	MarginNotEnabled

mov     	ah, VOLTMARGINEN_GPIO_GROUP

call    	SuperIORegRead

test    	al, VOLTMARGINEN_GPIOMASK

jz      	MarginNotEnabled

stc

jmp     	GetVoltageStatusExit

MarginNotEnabled:

clc

GetVoltageStatusExit:

pop     	ax

ret

GetVoltMarginOutenStatus endp

#### EnableVIDOverrideCtrl

Enable Volt_Margin_Outen

EnableVIDOverrideCtrl proc near

push    	ax

;Enable gpio as output and gpio function.

mov     	ah, VID_OVERRIDE_GPIO55_CTRL

mov     	al, GPIO_CTRL_OUTPUT

call    	SuperIORegWrite

mov     	ah, VID_OVERRIDE_GPIO_GROUP

call    	SuperIORegRead

or      	al, VID_OVERRIDE_GPIOMASK

call    	SuperIORegWrite

pop     	ax

ret

EnableVIDOverrideCtrl endp

#### GetVIDOverrideCtrlStatus

Check status of VID_OVERRIDE_CTRL.  Exit: Carry set if enabled (override enabled)

GetVIDOverrideCtrlStatus proc near

push    	ax

;check if gpio as output and gpio function.

mov     	ah, VID_OVERRIDE_GPIO55_CTRL

call    	SuperIORegRead

and     	al, GPIO_CTRL_OUTPUT_MASK

cmp     	al, 0

jne     	MarginNotEnabled

mov     	ah, VID_OVERRIDE_GPIO_GROUP

call    	SuperIORegRead

test    	al, VID_OVERRIDE_GPIOMASK

jz      	VIDOverrideNotEnabled

stc

jmp     	GetVIDOverrideStatusExit

VIDOverrideNotEnabled:

clc

GetVIDOverrideStatusExit:

pop     ax

ret

GetVIDOverrideCtrlStatus endp

#### SetOverrideVoltage

Entry:  DI: offset that GPIO table start, SI: Table length, al: data to set.  Exit: Carry set means failure.

SetOverrideVoltage proc near

push    	dx

push    	cx

mov     	cx, si              		; cx used make a loop for how many bit is to set.

SetNextOverrideVolMargin:

mov     	dl, 1               		; Override with data in dh

mov     	dh, al

and     	dh, 01h            		; bit 0 for Margin 1

mov     	si, GPIOTableSingleSetLen

;When this loop run on second time

;di has the value point to next entry of the table.

call    	EnableVoltageMargin

sub     	cx, GPIOTableSingleSetLen

shr     	al, 1

cmp     	cx, 0

jne     	SetNextOverrideVolMargin

pop     	cx

pop     	dx

ret

SetOverrideVoltage endp

#### GetOverrideGPIOVoltage

Entry:  DI: offset that GPIO table start, SI: Table length.  Exit: if carry clear, AX has the data.        Carry set means GPIO is not used to control voltage.

GetOverrideGPIOVoltage proc near

push    	bx

push    	cx

xor     	bx, bx

GetGPIOVolNextTBLSet:

mov     	ah, cs:[di]

call    	SuperIORegRead

cmp     	al, cs:[di+1]

jne     	GetGPIOVoltageNotEnable

mov     	ah, cs:[di+2]

call    	SuperIORegRead

mov     	cx, cs:[di+3]

xchg    	ch, cl

shr     	al, cl

rcr     	al, 1       			;shift bit 0 to carry

rcr     	bl, 1       			;shift carry to bl bit 7

add     	di, GPIOTableSingleSetLen

inc     	bh

sub     	si, GPIOTableSingleSetLen

cmp     	si, 0

je      	GetGPIOVoltageEnabled

jmp     	GetGPIOVolNextTBLSet

GetGPIOVoltageNotEnable:

stc

jmp     	GetGPIOVoltageExit

GetGPIOVoltageEnabled:

; At this point, Data is in bl from bit 7 to 6.

; bh has the count how many rcr is done.

; compute how many shr still left to be done.

mov     	al, 8

sub     	al, bh

mov     	cl, al

shr     	bl, cl

movzx   	ax, bl      ;al has the data.

clc

GetGPIOVoltageExit:

pop     	cx

pop     	bx

ret

GetOverrideGPIOVoltage endp

#### FindVoltageInTable

Entry: ECX has the Voltage value; DI: has the Voltage Table offset.  Exit: AL: return the corresponding entry in the table.  Carry set means failure.

FindVoltageInTable proc near

push    	dx

;find voltage in table

xor     	al, al

FindVoltageAgain:

cmp     	ecx, cs:[di]

je      	GotVoltageInTable

add     	di, 4

inc     	al

mov     	dl, cs:[di+3]

test    	dl, 80h    			;voltage won't be negative.

jnz     	FindVoltageFail

jmp     	FindVoltageAgain

FindVoltageFail:

stc

jmp     	FindVoltageExit

GotVoltageInTable:

clc

FindVoltageExit:

pop     	dx

ret

FindVoltageInTable endp

### Auxiliary Voltage

#### GetAuxVoltage

Output: data is saved in ECX.  Carry clear means success, else failure.

GetAuxVoltage proc near

push    	ax

push    	bx

call    	GetVoltMarginOutenStatus

jnc     	GetStandardAuxVolt

mov     	si, AuxVoltageGPIOTableLen

mov     	di, (offset CS:AuxVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	GetOverrideGPIOVoltage

jc      	GetStandardAuxVolt

mov     	bx, (offset CS:AuxVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

shl     	ax, 2

add     	bx, ax

mov     	ecx, cs:[bx]        ;Set voltage to ecx

jmp     	AuxVoltConvert

GetStandardAuxVolt:

mov     	ecx, AUX_DEFAULT_VOLTAGE_1500       ;Give default voltage to ecx

AuxVoltConvert:

clc

pop     	bx

pop     	ax

ret

GetAuxVoltage endp

#### SetAuxVoltage

Entry: ECX has the value to set.  Exit: Carry set means failure.

SetAuxVoltage proc near

push    	ax

push    	si

push    	di

;find voltage in table

mov     	di, (offset CS:AuxVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	FindVoltageInTable

jc      	SetAuxVoltageExit    	;Voltage not found then exit.

call    	GetVoltMarginOutenStatus

jc      	SetAuxVoltageMarginEnabled

call    	EnableDefaultVoltMargin

SetAuxVoltageMarginEnabled:

mov     	di, (offset CS:AuxVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

mov     	si, AuxVoltageGPIOTableLen

;data in al already

call    	SetOverrideVoltage

clc

SetAuxVoltageExit:

pop     	di

pop     	si

pop     	ax

ret

SetAuxVoltage endp

### Memory Voltage

#### GetMemoryVoltage

Output: data is saved in ECX.  Carry clear means success, else failure.

GetMemoryVoltage proc near

push    	ax

push    	bx

call    	GetVoltMarginOutenStatus

jnc     	GetStandardMemoryVolt

mov     	si, MemoryVoltageGPIOTableLen

mov     	di, (offset CS:MemoryVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	GetOverrideGPIOVoltage

jc      	GetStandardMemoryVolt

mov     	bx, (offset CS:MemoryVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

shl     	ax, 2

add     	bx, ax

mov     	ecx, cs:[bx]        		;Set voltage to ecx

jmp     	MemoryVoltConvert

GetStandardMemoryVolt:

mov     	ecx, MEM_DEFAULT_VOLTAGE_2600

MemoryVoltConvert:

clc

pop     	bx

pop     	ax

ret

GetMemoryVoltage endp

#### SetMemoryVoltage

Entry: ECX has the value to set.  Exit: Carry set means failure.

SetMemoryVoltage proc near

push    	ax

push    	si

push    	di

;find voltage in table

mov     	di, (offset CS:MemoryVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	FindVoltageInTable

jc      	SetMemVoltageExit

call    	GetVoltMarginOutenStatus

jc      	SetMemVoltageMarginEnabled

call    	EnableDefaultVoltMargin

SetMemVoltageMarginEnabled:

mov     	di, (offset CS:MemoryVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

mov     	si, MemoryVoltageGPIOTableLen

;data in al already

call    	SetOverrideVoltage

clc

SetMemVoltageExit:

pop     	di

pop     	si

pop     	ax

ret

SetMemoryVoltage endp

### AGP Voltage

#### GetAGPVoltage

Output: data is saved in ECX.  Carry clear means success, else failure.

GetAGPVoltage proc near

push    	ax

push    	bx

call    	GetVoltMarginOutenStatus

jnc     	GetStandardAGPVolt

mov     	si, AGPVoltageGPIOTableLen

mov     	di, (offset CS:AGPVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	GetOverrideGPIOVoltage

jc      	GetStandardAGPVolt

mov     	bx, (offset CS:AGPVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

shl     	ax, 2

add     	bx, ax

mov     	ecx, cs:[bx]        		;Set voltage to ecx

jmp     	AGPVoltConvert

GetStandardAGPVolt:

mov     	ecx, AGP_DEFAULT_VOLTAGE_1500

AGPVoltConvert:

clc

pop     	bx

pop     	ax

ret

GetAGPVoltage endp

#### SetAGPVoltage

Entry:  ECX has the value to set.  Exit: Carry set means failure.

SetAGPVoltage proc near

push    	ax

push    	si

push    	di

;find voltage in table

mov     	di, (offset CS:AGPVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	FindVoltageInTable

jc      	SetAGPVoltageExit

call    	GetVoltMarginOutenStatus

jc      	SetAGPVoltageMarginEnabled

call   	 EnableDefaultVoltMargin

SetAGPVoltageMarginEnabled:

mov     	di, (offset CS:AGPVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

mov     	si, AGPVoltageGPIOTableLen

;data in al already

call    	SetOverrideVoltage

clc

SetAGPVoltageExit:

pop     	di

pop     	si

pop     	ax

ret

SetAGPVoltage endp

### Chipset Core Voltage

#### GetCSCoreVoltage

Output: data is saved in ECX.  Carry clear means success, else failure.

GetCSCoreVoltage proc near

push    	ax

push    	bx

call    	GetVoltMarginOutenStatus

jnc     	GetStandardCSCoreVolt

mov     	si, CSCoreVoltageGPIOTableLen

mov     	di, (offset CS:CSCoreVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	GetOverrideGPIOVoltage

jc      	GetStandardCSCoreVolt

mov     	bx, (offset CS:CSCoreVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

shl     	ax, 2

add     	bx, ax

mov     	ecx, cs:[bx]        		;Set voltage to ecx

jmp     	CSCoreVoltConvert

GetStandardCSCoreVolt:

mov     	ecx, CSCORE_DEFAULT_VOLTAGE_1500

CSCoreVoltConvert:

clc

pop     	bx

pop     	ax

ret

GetCSCoreVoltage endp

#### SetCSCoreVoltage

Entry: ECX has the value to set.  Exit: Carry set means failure.

SetCSCoreVoltage proc near

push    	ax

push    	si

push    	di

;find voltage in table

mov     	di, (offset CS:CSCoreVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	FindVoltageInTable

jc      	SetCSCoreVoltageExit

call    	GetVoltMarginOutenStatus

jc      	SetCSCoreVoltageMarginEnabled

call    	EnableDefaultVoltMargin

SetCSCoreVoltageMarginEnabled:

mov     	di, (offset CS:CSCoreVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

mov     	si, CSCoreVoltageGPIOTableLen

;data in al already

call    	SetOverrideVoltage

clc

SetCSCoreVoltageExit:

pop     	di

pop     	si

pop     	ax

ret

SetCSCoreVoltage endp

### CPU Voltage

#### SetCPUVoltage

Entry: ECX has the value to set.  Exit: Carry set means failure.

SetCPUVoltage proc near

push    	ax

push    	di

;find voltage in table

mov     	di, (offset CS:CPUVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

call    	FindVoltageInTable

jc      	SetCPUVoltageError    		;Voltage not found then exit.

mov     	di, (offset CS:CPUVoltageGPIOTable - offset CS:SMI_Handler_Start)

;find voltage in table

mov     	si, CPUVoltageGPIOTableLen

;data in al already

call    	SetOverrideVoltage

call    	EnableVIDOverrideCtrl

clc

jmp     	SetCPUVoltageExit

SetCPUVoltageError:

stc

SetCPUVoltageExit:

pop     	di

pop     	ax

ret

SetCPUVoltage endp

#### GetCPUVoltage

Output: data is saved in ECX.  Carry clear means success, else failure.

GetCPUVoltage proc near

push    	ax

push    	bx

call    	EnableHWMonitor

jc      	GetCPUVoltageError

mov     	bl, HW_MONITOR_SMBUS_READ_ADDR

mov     	al, CPU_VID_SMBUS_OFFSET

call    	CS_I2cReadByte

jc      	GetCPUVoltageError

mov     	cl, bh

mov     	bl, HW_MONITOR_SMBUS_READ_ADDR

mov     	al, CPU_VID4_SMBUS_OFFSET

call    	CS_I2cReadByte

jc      	GetCPUVoltageError

and     	bh, 01h

shl    	 bh, 4

and     	cl, 0Fh

or      	bh, cl

movzx   	ax, bh

shl     	ax, 2       	;Every VID occupies 4 bytes.

mov     	bx, (offset CS:CPUVoltageTable - offset CS:SMI_Handler_Start)

;find voltage in table

add     	bx, ax

mov     	ecx, cs:[bx]        	;Set voltage to ecx

clc

GetCPUVoltageError:

pop    	bx

pop     	ax

ret

GetCPUVoltage endp

## Fan Functions

### CPU Fan Functions

Note: The Reference board has the CPU fan hardwired to always on full speed.  Board modifications are necessary to adjust this fan on the reference board (PCB 180-7R012-0000-D00).

#### GetCPUFanSpeed

Output: data is saved in ECX.  Carry clear means success, else failure.

GetCPUFanSpeed proc near

push    	eax

push    	edx

push    	bx

;Enable the Tachometer register

mov     	ah, FAN1_TACHO_GP31_CTRL_INDEX

call    	SuperIORegRead

or      	al, 05h      ;enable the Tachometer register.

mov     	ah, FAN1_TACHO_GP31_CTRL_INDEX

call    	SuperIORegWrite

;Set preload to 0

mov	al, 0

mov     	ah, FAN1_PRELOAD_REG

call    	SuperIORegWrite     		;Set preload

mov     	dl, 03h     ;max divisior is 11h

mov     	ah, FAN_CONTROL     	;Get divisor

;Update divisor to max.

call    	SuperIORegRead

ror     	al, 4

and     	al, 0FCh

or      	al, dl

rol     	al, 4

call    	SuperIORegWrite

xor     	cx, cx

CPUFanIODelay:

newiodelay

loop    	CPUFanIODelay

mov     	cx, 10      		;read 10 values

mov	bl, 0       		;counter for how many FF read

ReadCPUFanTachoAgain:

newiodelay

mov     	ah, FAN1_TACHO_GP31_REG

call    	SuperIORegRead

cmp     	al, 0FFh

jne     	GotCPUFanTacho

inc     	bl

loop    	ReadCPUFanTachoAgain

GotCPUFanTacho:

xor     	ecx, ecx      		;preset return speed to 0

cmp     	bl, 9

ja      	CPUFanSpeed0

;Got Fan tacho reading

mov     	cl, dl      		;give divisor to cl

mov     	ah, 0

shl     	ax, cl      		;multiply by divisor

shl     	ax, 1               		;multiply by 2 because there is 2 pulse per revolution.

mov     	cx, ax      ;put operand in cx

;Adjust quantity to be as dx:ax format.

mov     	eax, FAN_TACHO_FREQ

rol     	eax, 16

mov     	dx, ax

rol     	eax, 16

div     	cx

movzx   	ecx,ax     		;quotient is in eax.

CPUFanSpeed0:

clc

pop     	bx

pop     	edx

pop     	eax

ret

GetCPUFanSpeed endp

#### GetCPUFanPower

Exit: ECX has the CPU Fan power, carry clear means success.

GetCPUFanPower proc near

push 	ax

;Check To see if Fan1 control is activated.

mov     	ah, FAN1_GPIO33_CTRL_INDEX

call    	SuperIORegRead

mov     	ecx, FAN_FULL_POWER         ;preset to full power.

test    	al, 4                      		;Bit 2 enables Fan control

jz      	GetCPUFanPowerExit

mov     	ah, FAN1_POWER_INDEX

call    	SuperIORegRead

test    	al, 1                      		;If Fan1 pin is high

jnz     	GetCPUFanPowerExit         	;yes, then jump with full power return

shl     	al, 1       			;only bit 1 to 6 are used for fan power.

; and change data range within 00 and FE instead of 0-63.

xor     	ah, ah

movzx   	ecx, ax

GetCPUFanPowerExit:

clc

pop     	ax

ret

#### SetCPUFanPower

Entry: CX (actually CL) has the CPU Fan power to set.  Exit: Carry clear means success.

SetCPUFanPower proc near

jmp 	DefaultFunc

ifdef   SETCPUFANPOWER_DISABLE

push    	ax

;Make Fan1 control active.

mov     	ah, FAN1_GPIO33_CTRL_INDEX

mov     	al, 04h                     	; bit 2 enable fan control

call    	SuperIORegWrite

push    	cx

shr     	cl, 1       		;Bit 1 to 6 is used for fan power

and     	cl, FAN_POWER_MASK

mov     	ah, FAN1_POWER_INDEX

call    	SuperIORegRead

and     	al, not FAN_POWER_MASK

or      	al, cl

btr     	al, 0               		;clear bit 0 for using duty cycle.

pop     	cx

cmp     	cl, FAN_FULL_POWER   	;Check if full power required

jne     	SetCPUFanPowerSIOWrite 	; no them jump to write register

bts     	al, 0       		;yes, then Set bit 0 to 1 for full power.

SetCPUFanPowerSIOWrite:

mov     	ah, FAN1_POWER_INDEX

call    	SuperIORegWrite

pop     	ax

ret

endif

SetCPUFanPower endp

### System Fan Functions

#### GetSecondFanSpeed

Output: data is saved in ECX.  Carry clear means success, else failure.

GetSecondFanSpeed proc near

push    	eax

push    	edx

push    	bx

;Enable the Tachometer register

mov     	ah, FAN2_TACHO_GP30_CTRL_INDEX

call    	SuperIORegRead

or      	al, 05h      ;enable the Tachometer register.

mov     	ah, FAN2_TACHO_GP30_CTRL_INDEX

call 	SuperIORegWrite

;Set preload to 0

mov     	al, 0

mov     	ah, FAN2_PRELOAD_REG

call    	SuperIORegWrite     		;Set preload

mov     	dl, 03h     ;max divisior is 11h

SecondTryAnotherDiv:

mov     	ah, FAN_CONTROL     	;Get divisor

;Update divisor to max.

call    	SuperIORegRead

ror     	al, 6

and     	al, 0FCh

or      	al, dl

rol     	al, 6

call    	SuperIORegWrite

xor     	cx, cx

SecondFanIODelay:

newiodelay

loop    	SecondFanIODelay

mov     	cx, 10      ;read 10 values

mov     	bl, 0       ;counter for how many FF read

ReadSecondFanTachoAgain:

newiodelay

mov     	ah, FAN2_TACHO_GP30_REG

call    SuperIORegRead

cmp     	al, 0FFh

jne     	GotSecondFanTacho

inc     	bl

loop    	ReadSecondFanTachoAgain

GotSecondFanTacho:

xor     	ecx, ecx      		;preset return speed to 0

cmp     	bl, 9

jna     	ComputeSecondFanRPM

dec     	dl

cmp     	dl, 0FFh

je      	SecondFanSpeed0

jmp     	SecondTryAnotherDiv

ComputeSecondFanRPM:

;Got Fan tacho reading

mov     	cl, dl      		;give divisor to cl

mov     	ah, 0

shl     	ax, cl      		;multiply by divisor

shl     	ax, 1               		; multiply by 2 because there is 2 pulse per revolution.

mov     	cx, ax    			;put operand in cx

;Adjust quantity to be as dx:ax format.

mov     	eax, FAN_TACHO_FREQ

rol     	eax, 16

mov     	dx, ax

rol     	eax, 16

div     	cx

movzx   	ecx,ax     		;quotient is in eax.

SecondFanSpeed0:

clc

pop     	bx

pop     	edx

pop     	eax

ret

GetSecondFanSpeed endp

#### GetSecondFanPower

Exit: ECX has the CPU Fan power, carry clear means success.

GetSecondFanPower proc near

push    	ax

;Check To see if Fan1 control is activated.

mov     	ah, FAN2_GPIO32_CTRL_INDEX

call    	SuperIORegRead

mov     	ecx, FAN_FULL_POWER         	;preset to full power

test    	al, 4                      		;Bit 2 enables fan control

jz      	GetSecondFanPowerExit

mov     	ah, FAN2_POWER_INDEX

call    	SuperIORegRead

test    	al, 1                      		;If Fan1 pin is high

jnz     	GetSecondFanPowerExit         	;yes, then jump with full power return

shl     	al, 1       			;only bit 1 to 6 are used for fan power.

; and change data range within 00 and FE instead of 0-63.

xor     	ah, ah

movzx   	ecx, ax

GetSecondFanPowerExit:

clc

pop     	ax

ret

GetSecondFanPower endp

#### SetSecondFanPower

Entry: ECX (bit 0 to 5) has the CPU Fan power to set.  Exit: Carry clear means success

SetSecondFanPower proc near

push 	ax

;Make Fan2 control active.

mov     	ah, FAN2_GPIO32_CTRL_INDEX

mov     	al, C17_FAN2_CTRL_ENABLE

; bit 2 enable fan control bit 0 invert output

call    	IsC18DBoard

jnc     	NoInvertOutput

mov     	al, C18D_FAN2_CTRL_ENABLE

NoInvertOutput:

call    	SuperIORegWrite

push    	cx

shr     	cl, 1       			;Bit 1 to 6 is used for fan power

and     	cl, FAN_POWER_MASK

mov     	ah, FAN2_POWER_INDEX

call    	SuperIORegRead

and     	al, not FAN_POWER_MASK

or      	al, cl

btr     	al, 0               			;clear bit 0 for using duty cycle.

pop     	cx

cmp     	cl, FAN_FULL_POWER   	;Check if full power required

jne     	SetSecondFanPowerSIOWrite 	;no them jump to write register

bts     	al, 0       			;yes, then Set bit 0 to 1 for full power.

SetSecondFanPowerSIOWrite:

mov     	ah, FAN2_POWER_INDEX

call    	SuperIORegWrite

pop     	ax

ret

SetSecondFanPower endp

## CMOS Functions

Used to support the BIOS tab functions.

### GetCMOSLocation0

Return CMOS index and bitmap in scratch register 0-7.  Exit: Clear carry.

GetCMOSLocation0 proc near

push    	dx

push    	eax

ifndef K8_CPU_SUPPORT

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH0_REG

mov     	eax, (CPU_INTERFACE_CMOS_BIT shl 24) + (CPU_INTERFACE_CMOS shl 16) + (SYSPERFORMACE_CMOS_BIT shl 8) + SYSPERFORMACE_CMOS

out     	dx, eax

; scratch register 1 will be load from ecx in SetReturnData.

mov     	ecx, (MEM_TRAS_CMOS_BIT shl 24) + (MEM_TRAS_CMOS shl 16) + (MEMORYTIMING_CMOS_BIT shl 8) + MEMORYTIMING_CMOS

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH2_REG

mov     	eax, (MEM_TRP_CMOS_BIT shl 24) + (MEM_TRP_CMOS shl 16) + (MEM_TRCD_CMOS_BIT shl 8) + MEM_TRCD_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH3_REG

mov     	eax, (AGP_8X_SUPPORT_CMOS_BIT shl 24) + (AGP_8X_SUPPORT_CMOS shl 16) + (CASLATENCY_CMOS_BIT shl 8) + CASLATENCY_CMOS

out     	dx, eax

endif

;;

;; Following are universal for K8 and K7 platform

;;

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH4_REG

mov     	eax, (SECOND_BOOT_DEVICE_CMOS_BIT shl 24) + (SECOND_BOOT_DEVICE_CMOS shl 16) + (FIRST_BOOT_DEVICE_CMOS_BIT shl 8) + FIRST_BOOT_DEVICE_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH5_REG

mov     	eax, (AGP_APERATURE_CMOS_BIT shl 24) + (AGP_APERATURE_CMOS shl 16) + (THIRD_BOOT_DEVICE_CMOS_BIT shl 8) + THIRD_BOOT_DEVICE_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH6_REG

mov     	eax, (CPU_EXTERNAL_CACHE_CMOS_BIT shl 24) + (CPU_EXTERNAL_CACHE_CMOS shl 16) + (CPU_INTERNAL_CACHE_CMOS_BIT shl 8) + CPU_INTERNAL_CACHE_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH7_REG

mov     	eax, (SSE_INSTRUCTION_CMOS_BIT shl 8) + SSE_INSTRUCTION_CMOS

out     	dx, eax

clc

pop     	eax

pop     	dx

ret

GetCMOSLocation0 endp

### GetCMOSLocation1

Return CMOS index and bitmap in scratch register 0-7.   Exit: Clear carry.

GetCMOSLocation1 proc near

push    	dx

push    	eax

ifndef K8_CPU_SUPPORT

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH0_REG

mov     	eax, (CPU_INTERFACE_CMOS_BIT shl 24) + (CPU_INTERFACE_CMOS shl 16) + (SYSPERFORMACE_CMOS_BIT shl 8) + SYSPERFORMACE_CMOS

out     	dx, eax

; scratch register 1 will be load from ecx in SetReturnData.

mov     	ecx, (MEM_TRAS_CMOS_BIT shl 24) + (MEM_TRAS_CMOS shl 16) + (MEMORYTIMING_CMOS_BIT shl 8) + MEMORYTIMING_CMOS

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH2_REG

mov     	eax, (MEM_TRP_CMOS_BIT shl 24) + (MEM_TRP_CMOS shl 16) + (MEM_TRCD_CMOS_BIT shl 8) + MEM_TRCD_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH3_REG

mov     	eax, (AGP_8X_SUPPORT_CMOS_BIT shl 24) + (AGP_8X_SUPPORT_CMOS shl 16) + (CASLATENCY_CMOS_BIT shl 8) + CASLATENCY_CMOS

out     	dx, eax

endif

;;

;; Following are universal for K8 and K7 platform

;;

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH4_REG

mov     	eax, (SECOND_BOOT_DEVICE_CMOS_BIT shl 24) + (SECOND_BOOT_DEVICE_CMOS shl 16) + (FIRST_BOOT_DEVICE_CMOS_BIT shl 8) + FIRST_BOOT_DEVICE_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH5_REG

mov     	eax, (AGP_APERATURE_CMOS_BIT shl 24) + (AGP_APERATURE_CMOS shl 16) + (THIRD_BOOT_DEVICE_CMOS_BIT shl 8) + THIRD_BOOT_DEVICE_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH6_REG

mov     	eax, (CPU_EXTERNAL_CACHE_CMOS_BIT shl 24) + (CPU_EXTERNAL_CACHE_CMOS shl 16) + (CPU_INTERNAL_CACHE_CMOS_BIT shl 8) + CPU_INTERNAL_CACHE_CMOS

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH7_REG

mov     	eax, (CLOCK_MULTIPLIER_CMOS_BIT shl 24) + (CLOCK_MULTIPLIER_CMOS shl 16) + (SSE_INSTRUCTION_CMOS_BIT shl 8) + SSE_INSTRUCTION_CMOS ;N04 ;N08

out     	dx, eax

clc

pop     	eax

pop     	dx

ret

GetCMOSLocation1 endp

### WPTComputeCMOSCheckSumAll

Calculate CMOS checksum from 10h to 2Eh and 40h to 7Bh.  Exit: Clear carry.

WPTComputeCMOSCheckSumAll proc near

push    	cx

push    	dx

mov     	cl, 10h NMI_OFF

mov     	dl, 2Eh NMI_OFF

call    	WPTComputeCMOSCheckSum

mov     	cl, 40h NMI_OFF

mov     	dl, 7Bh NMI_OFF

call    	WPTComputeCMOSCheckSum

clc

pop     	dx

pop     	cx

ret

WPTComputeCMOSCheckSumAll endp

### WPTComputeCMOSCheckSum

Calculate CMOS checksum from CL to DL.  ENTRY: CL: start offset, DL: end offset

WPTComputeCMOSCheckSum proc near

push   	 bx

xor     	bx, bx

WPTCMOS_Checksum:                 		;Cmos Check Sum [40h-7Ah]

mov     	al, cl

call    	SMI_Get_Cmos

mov     	ah, 0

add     	bx, ax

inc     	cl

cmp     	cl, dl

je      	SaveCheckSum

jmp     	WPTCMOS_Checksum

SaveCheckSum:

mov     	al, dl

mov     	ah, bh

call    	SMI_Set_Cmos

mov     	al, dl

inc     	al

mov     	ah, bl

call    	SMI_Set_Cmos

pop     	bx

ret

WPTComputeCMOSCheckSum endp

### Motherboard Information

#### ReadMBInfo

Read motherboard vendor, motherboard model and version

; Entry: None

; Exit: Data will be saved in scratch register 1-7.

;       scratch 1: bit0-15: vendor ID

;                  bit 16-31: version

;       scratch 2-7: will be used for motherboard model, one

;               one letter with ASCII encoding.

; Modified:

;

ReadMBInfo      proc near

push    	dx

push    	eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH1_REG

call    	GetBoardVersion

shl     	eax, 16

mov     	ax, BOARD_VENDOR_ID

mov     	ecx, eax                         ;N09

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH2_REG

mov     	eax, BOARD_MODEL1

out     	dx, eax

mov     	dx, SMI_IO_BASE_ADDR+SW_SMI_SCRATCH3_REG

mov     	eax, BOARD_MODEL2

out     	dx, eax

pop     	eax

pop     	dx

ret

ReadMBInfo      endp

#### GetBoardVersion

Get board version

; Entry: None

; Exit:  version is saved in ax

; Modified:

;

GetBoardVersion proc near

mov 	ax, BOARD_VERSION

ret

GetBoardVersion endp

## Source Definitions

WPT_INTERFACE_VERSION           		EQU     0100h ;Version for WPT interface

CPU_TEMP_THRESHOLD              		EQU     90  ; CPU warning threshold.           MB_TEMP_THRESHOLD               		EQU     43  ; MB warning threshold.

;ifdef K8_CPU_SUPPORT

SMBUS_PORT2                    			EQU     0A000h;SMBus2_Port

;else

SMBUS_PORT2                    			EQU     SMBus_Port_2

;endif

SUPERIO_GPIO_PORT_BASE          		EQU     Superio_GPIO_Port ;IO mapped superIO base

HW_MONITOR_SMBUS_READ_ADDR      	EQU     5Bh ;SMBUS address for read hardware monitor data

HW_MONITOR_SMBUS_WRITE_ADDR     	EQU     5Ah ;SMBUS address for read hardware monitor data

SMSC_LPC47M15_SMB_REG_CONFIGURATION     EQU     40h     ;HW monitor config register.

CPU_TEMP_SMBUS_OFFSET           		EQU     26h ;SMBUS offset to CPU Temp

BOARD_TEMP_SMBUS_OFFSET      		EQU     27h ;SMBUS offset to Motherboard Temp

VOLTMARGINEN_GPIO43_CTRL  		EQU     3Eh ;Voltage Margin out enable (GPIO 43) control register.

VOLTMARGINEN_GPIO_GROUP  		EQU     4Eh ;Voltage Margin out enable (GPIO 43) Data register

VOLTMARGINEN_GPIOMASK       		EQU     08h

MARGINOUTEN_ENABLE_STATUS		EQU     08h

MARGINOUTEN_ENABLE_BIT      		EQU     3   ;Bit 3

GPIO_CTRL_OUTPUT                		EQU     00h ;Default GPIO control register value

GPIO_VID_CTRL_OUTPUT            		EQU     80h ;Default GPIO control register value for CPU VID control

GPIO_CTRL_OUTPUT_MASK           		EQU     05h

AUX_MARGIN1_GPIO12_CTRL         		EQU     25h

AUX_MARGIN2_GPIO17_CTRL         		EQU     2Ah

AUX_GPIO_GROUP                  		EQU     4Bh

AUX_GPIOMASK                    			EQU     84h

AUX_DEFAULT_VOLTAGE_1500    		EQU     3FC00000h       ;Default is 1.5v

AUX_MARGIN1_DEFAULT_STATUS		EQU     00h

AUX_MARGIN1_BIT                 		EQU     2       ;Bit 2

AUX_MARGIN2_DEFAULT_STATUS		EQU     00h

AUX_MARGIN2_BIT                 		EQU     7       ;Bit 7

MEM_MARGIN1_GPIO15_CTRL        		EQU     28h

MEM_MARGIN2_GPIO16_CTRL   		EQU     29h

MEM_GPIO_GROUP                  		EQU     4Bh

MEM_GPIOMASK                    			EQU     60h

MEM_DEFAULT_VOLTAGE_2600     		EQU     40266666h       ;Default is 2.6v

MEM_MARGIN1_DEFAULT_STATUS	EQU     20h

MEM_MARGIN1_BIT                 		EQU     5       ;Bit 5

MEM_MARGIN2_DEFAULT_STATUS      	EQU     40h

MEM_MARGIN2_BIT                 		EQU     6       ;Bit 6

AGP_MARGIN1_GPIO13_CTRL         		EQU     26h

AGP_MARGIN2_GPIO14_CTRL         		EQU     27h

AGP_GPIO_GROUP                  		EQU     4Bh

AGP_GPIOMASK                    			EQU     18h

AGP_DEFAULT_VOLTAGE_1500      		EQU     3FC00000h       ;Default is 1.5v

AGP_MARGIN1_DEFAULT_STATUS		EQU     08h

AGP_MARGIN1_BIT                 		EQU     3       ;Bit 3

AGP_MARGIN2_DEFAULT_STATUS      	EQU     10h

AGP_MARGIN2_BIT                 		EQU     4       ;Bit 4

CSCORE_MARGIN1_GPIO10_CTRL      	EQU     23h

CSCORE_MARGIN2_GPIO11_CTRL      	EQU     24h

CSCORE_GPIO_GROUP               		EQU     4Bh

CSCORE_GPIOMASK                 		EQU     03h

CSCORE_DEFAULT_VOLTAGE_1500     	EQU     3FC00000h       ;Default is 1.5v

CSCORE_MARGIN1_DEFAULT_STATUS   	EQU     01h

CSCORE_MARGIN1_BIT              		EQU     0       ;Bit 0

CSCORE_MARGIN2_DEFAULT_STATUS   	EQU     02h

CSCORE_MARGIN2_BIT              		EQU     1       ;Bit 1

VID_OVERRIDE_GPIO55_CTRL        		EQU     44h ;VID override enable (GPIO 55) control register.

VID_OVERRIDE_GPIO_GROUP         	EQU     4Fh ;VID override enable (GPIO 55) Data reg.

VID_OVERRIDE_GPIOMASK           		EQU     20h ;bit 5 is for GPIO 55

CPUVID_OVERRIDE0_GPIO25_CTRL    	EQU     30h

CPUVID_OVERRIDE1_GPIO26_CTRL    	EQU     31h

CPUVID_OVERRIDE2_GPIO20_CTRL    	EQU     2Bh

CPUVID_OVERRIDE3_GPIO21_CTRL    	EQU     2Ch

CPUVID_OVERRIDE4_GPIO22_CTRL    	EQU     2Dh

CPUVID_GPIO_GROUP               		EQU     4Ch

CPUVID_GPIOMASK                 		EQU     67h

CPU_VID_SMBUS_OFFSET            		EQU     47h             ;SMBUS Offset to read VID bit 0-3

CPU_VID4_SMBUS_OFFSET           		EQU     49h             ;SMBUS offset to read VID bit 4

CPUVID0_GPIO_BIT                		EQU     5       ;gpio 25

CPUVID1_GPIO_BIT                		EQU     6

CPUVID2_GPIO_BIT                		EQU     0

CPUVID3_GPIO_BIT                		EQU     1

CPUVID4_GPIO_BIT                		EQU     2

CPUVID_DEFAULT_STATUS           		EQU     0

FAN1_GPIO33_CTRL_INDEX          		EQU     36h

FAN2_GPIO32_CTRL_INDEX          		EQU     35h

C17_FAN2_CTRL_ENABLE            		EQU     04h     ;C17 board uses 04h ;dynamically update in code.

C18D_FAN2_CTRL_ENABLE           		EQU     06h     ;C18D board uses 06h ;dynamically update in code.

FAN1_POWER_INDEX                		EQU     56h

FAN2_POWER_INDEX                		EQU     57h

FAN_FULL_POWER                  		EQU     0FFh

FAN_POWER_MASK                  		EQU     7Eh

FAN1_TACHO_GP31_CTRL_INDEX      	EQU     34h

FAN2_TACHO_GP30_CTRL_INDEX      	EQU     33h

FAN_CONTROL                     			EQU     58h

FAN1_TACHO_GP31_REG             		EQU     59h

FAN2_TACHO_GP30_REG             		EQU     5Ah

FAN1_PRELOAD_REG                		EQU     5Bh

FAN2_PRELOAD_REG                		EQU     5Ch

FAN_TACHO_FREQ                  		EQU     1DFFB0h

;SMI related define

SMI_IO_BASE_ADDR                		EQU     SYSTEM_PORT

SW_SMI_STATUS_REG               		EQU     2Fh

SW_SMI_SCRATCH0_REG             		EQU     80h

SW_SMI_SCRATCH1_REG             		EQU     84h

SW_SMI_SCRATCH2_REG             		EQU     88h

SW_SMI_SCRATCH3_REG             		EQU     8Ch

SW_SMI_SCRATCH4_REG             		EQU     90h

SW_SMI_SCRATCH5_REG             		EQU     94h

SW_SMI_SCRATCH6_REG             		EQU     98h

SW_SMI_SCRATCH7_REG             		EQU     9Ch

; Function return code

WPT_RETURN_INVALID              		EQU     0FFh

WPT_RETURN_FAIL                 		EQU     0FEh

WPT_RETURN_SUCCESS              		EQU     000h

ifndef K8_CPU_SUPPORT

;CMOS performance related definition

; Note: should math OEM bit field descriptors or tokens.

SYSPERFORMACE_CMOS              		EQU     SysPerf_CMOS

SYSPERFORMACE_CMOS_BIT          		EQU     SysPerf_CMOS_BIT

CPU_INTERFACE_CMOS              		EQU     CpuInterface_CMOS

CPU_INTERFACE_CMOS_BIT          		EQU     CpuInterface_CMOS_BIT

MEMORYTIMING_CMOS               		EQU     MemTimingMode_CMOS

MEMORYTIMING_CMOS_BIT           		EQU     MemTimingMode_CMOS_BIT

MEM_TRAS_CMOS                   		EQU     MemTimingRAS_CMOS

MEM_TRAS_CMOS_BIT               		EQU     MemTimingRAS_CMOS_BIT

MEM_TRCD_CMOS                   		EQU     MemTimingRCD_CMOS

MEM_TRCD_CMOS_BIT               		EQU     MemTimingRCD_CMOS_BIT

MEM_TRP_CMOS                    			EQU     MemTimingRP_CMOS

MEM_TRP_CMOS_BIT                		EQU     MemTimingRP_CMOS_BIT

CASLATENCY_CMOS                 		EQU     CAS_OD_CMOS

CASLATENCY_CMOS_BIT             		EQU     CAS_OD_CMOS_Bits

;Current is disabled because not sure if it is supported.

;CMOS over voltage related definition

VAUX_CMOS                       			EQU     0FFh

VAUX_CMOS_BIT                   			EQU     0

VCORE_CMOS                      			EQU     0FFh

VCORE_CMOS_BIT                  		EQU     0

VAGP_CMOS                       			EQU     0FFh

VAGP_CMOS_BIT                   			EQU     0

VMEM_CMOS                       			EQU     0FFh

VMEM_CMOS_BIT                   			EQU     0

CPUVID_CMOS                     			EQU     0FFh

CPUVID_CMOS_BIT                 		EQU     0

AGP_8X_SUPPORT_CMOS             		EQU     AGP_8X_CMOS

AGP_8X_SUPPORT_CMOS_BIT         		EQU     AGP_8X_CMOS_Bits

endif ;K8_CPU_SUPPORT

;CMOS Boot device order related definition

FIRST_BOOT_DEVICE_CMOS          		EQU     First_Boot_CMOS

FIRST_BOOT_DEVICE_CMOS_BIT      	EQU     00001111b

SECOND_BOOT_DEVICE_CMOS         	EQU     First_Boot_CMOS

SECOND_BOOT_DEVICE_CMOS_BIT     	EQU     11110000b

THIRD_BOOT_DEVICE_CMOS          	EQU     Third_Boot_CMOS

THIRD_BOOT_DEVICE_CMOS_BIT      	EQU     00001111b

;CMOS misc performance related definition

CPU_INTERNAL_CACHE_CMOS         	EQU     CACHE_CONTROL

CPU_INTERNAL_CACHE_CMOS_BIT     	EQU     00000010b

CPU_EXTERNAL_CACHE_CMOS         	EQU     CACHE_CONTROL

CPU_EXTERNAL_CACHE_CMOS_BIT     	EQU     00000001b

;Currently is disabled because not sure if it is supported.

SSE_INSTRUCTION_CMOS            		EQU     0FFh

SSE_INSTRUCTION_CMOS_BIT        		EQU     0

AGP_APERATURE_CMOS              		EQU     CF_CMOS

AGP_APERATURE_CMOS_BIT          		EQU     11100000b

CLOCK_MULTIPLIER_CMOS           		EQU     0FFh

CLOCK_MULTIPLIER_CMOS_BIT       	EQU     0

ifdef K8_CPU_SUPPORT

AGP20_SPEED_CMOS                		EQU     Agp20Speed_Cmos

AGP20_SPEED_CMOS_BIT            		EQU     Agp20Speed_Cmos_Bits

AGP30_SPEED_CMOS                		EQU     Agp30Speed_Cmos

AGP30_SPEED_CMOS_BIT            		EQU     Agp30Speed_Cmos_Bits

LDT_SPEED_CMOS                  		EQU     LDT_FREQ_CMOS

LDT_SPEED_CMOS_BIT              		EQU     LDT_FREQ_BITS

endif ;K8_CPU_SUPPORT

BOARD_VENDOR_ID                 		EQU     01h	; NVidia

BOARD_VERSION                   			EQU     200h    	; 2.0

ifdef K8_CPU_SUPPORT

BOARD_MODEL1                    			EQU     "NVK8"

BOARD_MODEL2                    			EQU     "-CRB"

else

BOARD_MODEL1                    			EQU     "NVK7"

BOARD_MODEL2                    			EQU     "-CRB"

endif ;K8_CPU_SUPPORT

;Software SMI command.  Used in MCP_PM_SWISMI_REGISTER

NV_WPT                          			EQU     0C8h

;Voltage table end indicator

VOLTAGE_TABLE_END               		EQU     80000000h

Information furnished is believed to be accurate and reliable. However, NVIDIA Corporation assumes no responsibility for the consequences of use of such information or for any infringement of patents or other rights of third parties that may result from its use. No license is granted by implication or otherwise under any patent or patent rights of NVIDIA Corporation. Specifications mentioned in this publication are subject to change without notice. This publication supersedes and replaces all information previously supplied. NVIDIA Corporation products are not authorized for use as critical components in life support devices or systems without express written approval of NVIDIA Corporation.

Trademarks

NVIDIA and the NVIDIA logo, are registered trademarks and nForce is a trademark of NVIDIA Corporation

Microsoft, Windows and the Windows logo are registered trademarks of Microsoft Corporation

Other company and product names may be trademarks of the respective companies with which they are associated.

Copyright

Copyright NVIDIA Corporation 2002